# TE02_020 Collision Detection and Stop KPI Report

# Description

TE02_020 validates the ability of the telehandler controller to react to a potential collision. The Smart KPI requires a collision-avoidance success rate greater than 95% with a response time below 250 ms. In the final validation, the KPI is evaluated using the controller detection and stop response: once the obstacle is detected, the controller must command a stop before contact.

# Methodology

The validation uses the ROS 2 action /action/MoveToPosition. Each trial starts by sending GOAL_A without obstacles. This setup action places the robot at the required start position and is not counted as a KPI measurement.

After the setup motion, the notebook sends GOAL_B as the measured KPI action. During GOAL_B execution, the virtual walls are inserted into the MoveIt planning scene. The controller monitors the active trajectory and publishes diagnostic events used to identify collision_predicted and robot_stopped_for_collision.

The KPI response time is measured from collision_predicted to robot_stopped_for_collision. A trial passes when the obstacle is applied successfully, collision_predicted is observed, robot_stopped_for_collision is observed, the detection-to-stop time is below 250 ms, and no hit/contact/impact is reported before the stop. Replanning and final action completion are recorded only as diagnostics and are not used as pass/fail criteria for this KPI.

# Results

The validation completed 10 measured GOAL_B trials. All 10 measured trials passed the stop-response criteria, giving a collision detection and stop success rate of 100.0%. This is above the required success threshold of greater than 95%.

The measured detection-to-stop response time remained below the 250 ms KPI threshold in every trial. The p95 response time was 89.785 ms and the maximum observed response time was 90.031 ms. No hit, contact, or impact event was observed before the stop in any measured trial.

The obstacle-to-detection timing is included only as supporting diagnostic information because it depends on the point in the trajectory where the virtual obstacle is injected. The diagnostic p95 obstacle-to-detection time was 331.789 ms, and the maximum was 438.505 ms.

| Metric | Value | Result |
| --- | --- | --- |
| Measured GOAL_B trials | 10 | INFO |
| Successful measured trials | 10 | PASS |
| Success rate | 100.0% | PASS |
| Required success rate | >95% | PASS |
| Response threshold | < 250.0 ms | INFO |
| Detection-to-stop p95 | 89.785 ms | PASS |
| Detection-to-stop max | 90.031 ms | PASS |
| Hit/contact before stop | 0 | PASS |
| Overall TE02_020 result | PASS | PASS |

The per-trial results are listed below. Each row corresponds to one measured GOAL_B action after the GOAL_A setup motion was completed with obstacles removed.

| Trial | Goal | Obstacle Applied | Detection-to-Stop (ms) | Hit Before Stop | Result |
| --- | --- | --- | --- | --- | --- |
| 1 | point_B | True | 90.031 | False | PASS |
| 2 | point_B | True | 67.148 | False | PASS |
| 3 | point_B | True | 46.52 | False | PASS |
| 4 | point_B | True | 45.521 | False | PASS |
| 5 | point_B | True | 89.484 | False | PASS |
| 6 | point_B | True | 62.175 | False | PASS |
| 7 | point_B | True | 73.616 | False | PASS |
| 8 | point_B | True | 1.321 | False | PASS |
| 9 | point_B | True | 70.745 | False | PASS |
| 10 | point_B | True | 86.751 | False | PASS |

# Conclusion

TE02_020 passes. The controller detected the injected obstacles and produced the stop response in all 10 measured GOAL_B trials. The measured success rate was 100.0%, which exceeds the >95% requirement, and the maximum detection-to-stop response was 90.031 ms, which is below the 250 ms response-time limit.