# TEO2-026: ROS Action Request Reception and Processing

# Description

This KPI evaluates whether the system successfully receives and processes ROS action requests with a success rate greater than 90%. The validation focuses on the communication path of the ROS action interface, not on the internal success criteria of the action behavior.

The action server under test is /action/effector_controller, using action type telehandler_moveit/action/EffectorController.

# Methodology

The validation uses a ROS 2 action client to send repeated action requests to the /action/effector_controller action server. For each request, the notebook checks whether the action server is available, whether the goal is accepted, and whether a result response is received before timeout.

A request counts as a successful action communication transaction when the server is available, the goal is accepted, and the client receives a result response. The action result field success is recorded as diagnostic information only and does not define the KPI pass/fail result.

The communication success rate is calculated as:

communication_success_rate = communication_successful_requests / total_requests * 100

The KPI passes when the communication success rate is greater than 90%.

The test sends 30 action requests with the following configured goal parameters:

| Goal field | Value |
| --- | --- |
| frame_target | arm_fork_link |
| frame_reference | basket_link |
| height_distance | 0.0 |
| angle_degrees | -10.0 |
| max_velocity | 1.0 |
| auto_adjust | False |

# Results

The validation sent 30 action requests to /action/effector_controller. The action server was available for all requests, all goals were accepted, and all result responses were received. This produced a communication success rate of 100.0%.

| Metric | Result | Evaluation |
| --- | --- | --- |
| Total action requests | 30 | INFO |
| Server available requests | 30 / 30 | PASS |
| Accepted requests | 30 / 30 | PASS |
| Result responses received | 30 / 30 | PASS |
| Communication successful requests | 30 / 30 | PASS |
| Communication success rate | 100.0% | PASS |
| Required threshold | > 90.0% | PASS |
| 95th percentile response latency | 901.78 ms | INFO |
| Overall TE02-026 result | PASS | PASS |

The action feedback log contains 404 feedback messages, providing additional evidence that the action server receives and processes goals during execution. The feedback reports the active frame as basket_link_to_armFork_joint and includes planning/executing state updates during the action requests.

The action result diagnostic field also reported success for 30/30 requests. This value is recorded for traceability, but the KPI evaluation is based on action communication completion: server availability, goal acceptance, and result-response reception.

# Conclusion

The validation demonstrates that the /action/effector_controller ROS action interface successfully receives and processes action requests from a ROS 2 client. All 30 action requests completed the required communication transaction: the action server was available, the goals were accepted, and result responses were received.

The measured action communication success rate is 100.0%, which exceeds the required threshold of greater than 90%.

Based on the current validation results, TE02-026 is validated successfully.